ESPBanking System
(Free & Pro Editions)

Made by Dumasha
Email: dumasha.contact@gmail.com
Reddit Discussion: https://www.reddit.com/r/esp32projects/comments/1vgzaix/i_built_a_standalone_offline_banking_server/

# Pro Edition ($15)

Unlock the full power of ESPBanking for advanced deployments and professional setups. Features include advanced security (SSL/TLS), unlimited user slots, and encrypted database integration.

Purchase ESPBanking Pro Here: https://buy.polar.sh/polar_cl_0bKVXNPvRLxRmi2gwR9kSxV05FBdRlY7aQK0i4DmsnZ

# Free Edition (Lite)

The Free Edition provides a fully functional, standalone local application designed for hobbyists and makers. It features a capped 3-user system, basic transaction logic, and real-time ST7789 TFT display monitoring.

## Hardware & Pin Configuration (ST7789 SPI)

TFT_CS: GPIO 15

TFT_DC: GPIO 2

TFT_RST: GPIO 4

TFT_MOSI (SDA): GPIO 23

TFT_SCLK (SCL): GPIO 18

# How to Download esptool & Flash the Binary

esptool is the official Python utility used to flash firmware to ESP32 chips. Follow these steps to upload the Free Edition binary to your board.

## 1. Download & Install esptool

Open your command prompt or terminal and install the tool using Python:

py -m pip install esptool

## 2. Connect Your ESP32

Plug your ESP32 into your computer using a data-capable USB cable. Note the assigned COM port (e.g., COM3 on Windows, or /dev/ttyUSB0 on Linux/Mac). Put the board into download mode if required by holding the BOOT button, pressing RST, and releasing BOOT.

## 3. Erase Flash Memory (Recommended)

Wipe any old data to prevent boot loops. Replace COM3 with your actual port:

py -m esptool --chip esp32 --port COM3 erase_flash

## 4. Flash the Firmware Binary

If you are using a single merged binary file:

py -m esptool --chip esp32 --port COM3 --baud 921600 write_flash 0x0 espbanking_free_merged.bin

If you are using separate component binaries (Bootloader, Partitions, and App):

py -m esptool --chip esp32 --port COM3 --baud 921600 write_flash 0x1000 bootloader.bin 0x8000 partitions.bin 0x10000 espbanking_free.bin

## 5. Wi-Fi Provisioning & Startup

Open your Serial Monitor at 115200 baud. If no network credentials are saved, the ESP32 will host an Access Point named 'ESPBank-Local' (Password: 12345678). You can also provision it directly via the serial terminal by sending:

WIFI:YourSSID,YourPassword

Once connected, navigate to the assigned IP address shown on the TFT screen to access the dashboard.

# License & Copyright

© 2026 Dumasha. All Rights Reserved.

This software is the intellectual property of Dumasha. You are permitted to download and use the Free Edition for personal, non-commercial purposes.

Restrictions:

No Copying or Stealing: You may not copy, reverse-engineer, modify, distribute, or re-upload this code or binary as your own work.

No Commercial Use: You may not sell or monetize this software in any form.

Pro Edition: The Pro version is strictly for the original purchaser and cannot be shared or redistributed.

By downloading or using this software, you agree to these terms.